2011-CURRENT POLARIS RUSH ASSAULT INSTALLATION INSTRUCTIONS    011312

WARNING: When installing your airshocks, it is best to pressurize each airspring to 20psi (minimum) before installing. If the airspring becomes deflated anytime after installation, DO NOT lift the snowmobile’s chassis by hand or mechanically, this may cause the airspring’s piston or endcap to become unseated. Simply add air pressure to the deflated airspring(s) to raise the chassis, and then set each airspring to your desired pressure.  Never operate any airspring below 25 psi.

- Safely and securely lift and support the rear of sled.

- Remove the stock FT-Shock and the stock RT-Shock.

- Set the factory nuts and bolts aside as they will be used with this kit.

- The FT-Shock has the bushings in place from TeamFAST. Install the FT-Shock with the upper air fitting pointing out to the LH side of sled. The Nitrogen filler port on the lower shock eye will be on the upper side facing the rear of sled. Bolt into place with factory hardware and torque the bolts to the Polaris factory specifications.

- The RT-Shock has the bushings installed from TeamFAST. Install the RT-Shock with the 90 degree elbow on the upper shock eye on the bottom side and the hose exiting on the RH side of the sled. Bolt into place with factory hardware and torque the bolts to the Polaris factory specifications. See Photos.

- Route the S/S braided air line out the RH side of sled. Secure to frame tube with ties provided. See Photos.

- A starting air pressure in the FT-Shock is 40-45 PSI. A starting air pressure for the RT-Shock is 65-70 PSI. You will fine tune and tailor the air pressures to your riding preferences. Minimum air pressure for ASSAULT shocks is 25 PSI. Never run below 25 PSI.

SAFETY NOTICE:

As with tires on an automobile the owner of any air suspension equipped vehicle must validate that the suspension’s air pressure is sufficient for safe operation before allowing use of the air suspension equipped vehicle.   All rubber and plastic products including O-rings, hoses, rubber bladders and tires have some level of porosity which air can and will pass through over time.  In the purchase and or acceptance of this product the owners and users agree to and accept certain responsibilities.  These include making certain that a static bounce test and a slow safety check ride is regularly performed before using the vehicle to establish that the sled and air suspension are functioning properly.